INTRODUCTION

Schneider Electric make PLC and SCADA Systems (29 nos.) has been provided for process control at Turamdih Mill of UCIL. All existing SCADA systems are running on Microsoft Windows 2000 and Windows XP. The license keys of the SCADA Software are in the form of hard keys with parallel port connector. Maintenance of SCADA system is becoming difficult because present Operating System and Parallel Ports are now obsolete.

UCIL intends to upgrade the SCADA system in phased manner by upgrading the SCADA Software, associated Personal Computers (HMI Stations) and Operating System. Proposed SCADA systems shall be compatible with the latest Windows operating system  and shall have license  in the form USB port Keys.

SCOPE OF WORK:

  - SCOPE of Supply.

    - Supply of SCADA Software of Vijeo Citect Latest version (in CD/DVD form)

    - SCADA license (in USB key form), device drivers, software patches and any other software that may be required for smooth execution of the work.

  - SCOPE of Services

    - CITECT project backup of all running projects and its conversion (Citect 6.0 to Vijeo Citect7.4).

    - Installation of new SCADA (Vijeo Citect 7.4 or latest) software in Desktop Computer having Windows 8.2/10)

    - Restoration of converted project in the upgraded SCADA System, system testing & necessary modifications, fixing of bugs, if any etc.

Present system configuration:

List of licenses to be upgraded:

| Sl. No. | PLANT AREA | LICENSE Ver. | LICENSE Type | Connection Type | License Serial No. |
| --- | --- | --- | --- | --- | --- |
| 01 | Fine Crushing | CITECT 6.1 | 5000 tag, SERVER | USB | 0479-09388 |
| 02 | Fine Crushing | CITECT 6.1 | 15000 tag, CLIENT | Parallel | 0479-09350 |
| 03 | Fine Crushing | CITECT 6.1 | 5000 tag, SERVER | USB | 0479-09387 |
| 04 | Fine Crushing | CITECT 6.1 | 15000 tag, CLIENT | Parallel | 0479-09352 |
| 05 | ION-X | CITECT 6.1 | 5000 tag, SERVER | Parallel | 0479-09386 |
| 06 | ION-X | VIJEO CITECT 7.1 | 15000 tag, SERVER | Parallel | 0479-81699 |
| 07 | PPN | CITECT 6.1 | 15000 tag, CLIENT | Parallel | 0479-09356 |
| 08 | Instrumentation | CITECT 6.1 | 15000 tag, CLIENT | Parallel | 0479-09354 |
| 09 | Neutral Thickener | VIJEO CITECT 7.0 | 500 tag, SERVER | USB | 0479-64473 |
| 10 | LSP | VIJEO CITECT 7.0 | 500 tag, SERVER | USB | 0479-64474 |

Price Format

| I. SUPPLY PART |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Name of Item | Quantity | Unit Price | Excise duty | Sales tax | Any other Taxes & duties) | Total Unit price (including all Taxes & duties) | Total Price |
| A | B | C | D | E | F | G | H=D+E+F+G | I=H * C |
|  | Supply of SCADA Software of Vijeo Citect, latest version (in CD/DVD form), its License (in USB key form), device drivers, software patches and any other software that may be required for license serial no.: |  |  |  |  |  |  |  |
| 1 | 0479-09388 | 1 |  |  |  |  |  |  |
| 2 | 0479-09350 | 1 |  |  |  |  |  |  |
| 3 | 0479-09387 | 1 |  |  |  |  |  |  |
| 4 | 0479-09352 | 1 |  |  |  |  |  |  |
| 5 | 0479-09386 | 1 |  |  |  |  |  |  |
| 6 | 0479-81699 | 1 |  |  |  |  |  |  |
| 7 | 0479-09356 | 1 |  |  |  |  |  |  |
| 8 | 0479-09354 | 1 |  |  |  |  |  |  |
| 9 | 0479-64473 | 1 |  |  |  |  |  |  |
| 10 | 0479-64474 | 1 |  |  |  |  |  |  |
| SERVICE PART |  |  |  |  |  |  |  |  |
| Sl. No. | Name of Service | Quantity | Unit Price | Service Tax |  | Any other Taxes & duties) | Total Unit price (including all Taxes & duties) | Total Price |
| A | B | C | D | E |  | F | G=D+E+F | H=G *C |
| 1 | Taking CITECT project backup of running projects, its conversion (Citect 6.0 to the supplied software), Installing Supplied SCADA (Vijeo Citect 7.4 or latest) in Desktop Computer (loaded with Windows 8.2/10), restoring the converted project in Supplied SCADA, system testing, modifying the project if required, fixing of bugs etc. | 1 lot |  |  |  |  |  |  |
| TOTAL PRICE (SUPPLY + SERVICE) in Figure Rs.: |  |  |  |  |  |  |  |  |
| TOTAL PRICE (SUPPLY + SERVICE) in Words Rs.: |  |  |  |  |  |  |  |  |

Special conditions of the work:

- Work related to report generation, PLC modification, and new SCADA development is excluded from the scope of this tender.

- Successful bidder may depute their service engineer at Turamdih Mill on mutually agreed dates for site related jobs.

- Successful bidder has to make their own arrangement for the lodging & boarding of their site engineer.

- Bidder shall record specific points related to pre-installation activity that may be required for successful execution of the project in their bid.

- Successful bidder has to sign a confidentiality agreement with UCIL. Vital information related to control logics, hardware configuration, SCADA projects of UCIL shall not be disclosed/ shared with third party.

- SCADA software shall be installed in Desktop computers of following specification: i7 processors, 4GB RAM, 500HDD, 1Gbps Ethernet & Windows 8 OR Windows 10 operating system. Bidder may propose better configuration of desktop computers, if necessary.

- UCIL has observed bit latching problem (during “PULSE” command in SCADA Vijeo Citect 7.1). Successful bidder has to ensure that above problem shall be resolved during up gradation process.

- Schneider Electric India Pvt. Limited or its authorised System Integrators only can participate in the present tender. Copy of valid ‘Authorised System Integrator’ certificate shall be submitted along with the offer.

- Bidder has to submit a declaration from Schneider Electric, that there is no bar in supplying SCADA system in Uranium Corporation India Ltd. which is under Department of Atomic Energy. In such case original PLC manufacturer has to supply & service during & after project execution including AMC directly to UCIL for a minimum period of 10 years.

General Terms and Conditions

- Nature of the tender

The tenderer should submit tender in Single Part in two different sealed envelopes .One for Technical bid and the other for price bid. The envelopes shall be clearly superscript as technical bid and price bid.

- Preparation of tender

- Before submission of the tender the tenderers are advised to make themselves fully conversant with the conditions of tendering, general conditions, special conditions, site conditions, specifications, schedules, drawings and all other relevant information’s so that no ambiguity may arise in this respect subsequent to the submission of the tender.

- It shall be the responsibility of the tenderer to request for any missing document. In absence of any such request the tenderer will be deemed to have received and read all documents.

- The tenderer shall submit his tender strictly in accordance with the tender specification and terms and conditions laid down in the tender document.

- Time of completion

The time of completion of the total job including supply, erection, commissioning and trial run of the total system is 3 (Three) Months from the date of issue purchase order.

- Total cost (i.e. cost of supply & services) as per price format shall be use to decide the Lowest (L-1) Bidder.

- Schneider Electric or its authorised system integrators can participate in the present tender.

- Security Deposit: Total amount of security deposit will be 5% of total value of the work. EMD amount will be converted in to security deposit and balance amount is to be paid either in cash or Bank Draft, in favor Uranium Corporation of India Limited, payable at State Bank Of India, Jaduguda Branch, Code No. 0227.  Successful bidder has to submit security deposit within 30 days after release of purchase order.

- Payment Terms

Supply:

80% payment along with taxes and duties with 30 days of receipt an acceptance of material of our central stores.

Balance 20 % after completion of erection & commissioning of system.

Services:

100% payment along with taxes and duties with 30 days after completion of erection & commissioning of system.

- Income tax shall be deducted as per I.T. Act. Income Tax & Work Contract Tax, if applicable, shall be deducted from your bills

- VALIDITY: Offers shall be valid for 120 days effective from the date of tender opening.

- The Purchaser may supply, on free of cost basis, electricity (415V, 3Ph) in controlled quantity for construction, erection and commissioning purposes at one point. The contractor shall have to make his own arrangement for distribution/ extension of power line at his own cost.

- The Contractor shall submit 03 (three) sets of drawings, documents, data, etc. for approval.

The remark /approval to the Contractor regarding the approval of drawings/ documents shall be conveyed by the Purchaser in reasonable time. However, Contractor may have to visit the Purchaser’s office at his own cost to explain, Co-ordinate & expedite the approval of his drawings/documents so that the work schedule can be strictly maintained. Purchaser may at their discretion decide to approve only General Arrangement (G.A.) drawings and balance leave to the tenderer.

Upon receipt of prints which shall be marked  “Approved” or “Approved except as noted, forward final drawings”, the Contractor shall furnish within a reasonable time three (03) additional prints and one (01) reproducible transparency (soft copy) after minor correction, if any, to the Purchaser.

On completion of erection, testing and commissioning, the Contractor shall furnish the “As built drawings”, in numbers as specified above. The “As built drawings” shall be prepared into consideration any changes (as approved by the Engineer) during erection and testing from the approved drawings.

- On arrival of material at site, the Contractor shall open the packing, cases, inspect the materials. Repair and replacement of material damaged or lost in transmit or at site should be done promptly.

- The Contractor shall supply, as per the programme, three (03) copies of erection, testing, operation and maintenance manuals to the Purchaser.

- No compensation shall be paid to the Contractor for temporary idling of their workers, officers and equipments, etc. for hooking/matching up of the works awarded, with the existing plant’s equipment which may arise due to non-availability of site, equipments facilities etc. However best efforts shall be put up by the Purchaser, without disturbing production of the operating plant, to extend all help and facilities at site to the Contractor for carrying out the works satisfactorily without any loss of time/man/ hour etc.

- During erection, testing and commissioning, the Contractor shall be responsible for adequate training of the Purchaser’s personnel so as to enable them to carry out operation and maintenance of the plant machinery under specification.

- The sequence of tests of individual equipment shall be mutually agreed to between the Engineer and the Contractor.

- Reimbursement for variation in Taxes & Duties

The Contractor will be paid at actual against production of documentary evidence for any variation in statutory taxes, duties etc. or any imposition of new Governmental taxes /duties during the Contract execution etc. period. For this purpose, the bidder indicate clearly in his price bid the details of taxes and duties considered for all those major items for which he intends to claim for such reimbursement.

- Defect Liability

As soon as the work have been completed in accordance with the contract and handed over to UCIL after completion of performance and other tests relevant thereof, UCIL will take over the System from the Contractor and will issue a taking over certificate, certifying date on which the works have been completed. The System shall be guaranteed for any defects, malfunctioning, poor performance, non-attainment of capacity, for a period of 12 months from date of issue of taking over certificate and performance bond submitted by the Contractor will remain valid till this period.

- Delay in Completion

If the Contractor fails to complete the works and tests within the time of completion the Purchaser shall be entitled to impose liquidated damage a sum @1/2% of contract sum for each complete week between the time for completion and actual date of completion, maximum up to 05% of the contract value.

- Total Liability:

The maximum limits for liability for performance is 10% of total contract value, for liquidated damage 05% of the total contract value and total liabilities on these two accounts combined to maximum 10% of total contract value.

- Any further clarification is required; party may come to see the present system before quoting.

- You shall maintain all safety precautions during execution of work. You should

- Bidder has to submit all techno-commercial deviation from the tender in a separate sheet.

- Medical Care

The Contractor shall be fully responsible for first aid and emergency medical treatment to his employee at site. The Contractor at site shall make necessary arrangement for this purpose. In serious cases, medical facilities of UCIL may be available to the Contractor on chargeable basis.

The medical supervision of Contractor over his employee shall include anti-malaria measures, vaccination against small pox, and inoculations against cholera, typhoid and other infectious diseases. Employees suffering from infectious disease shall be removed as soon as detected. If any case of infectious disease be discovered amongst the employees, it must at once be reported to the Engineer. The Contractor shall abide by the provisions of the Employees State Insurance Scheme as applicable.