The material description has been modified as follows.

Amended Material Description

For: Filter bags, (Length=5m), Dia=160mm, Non woven polyester needle felt. Technical specifications Annexure-A Read: Filter bags, (Length=5m), Dia=160mm, Non woven polyester needle felt. Technical specifications Annexure-A1

Annexure-A1

SPECIFICATION OF BAG FOR DE SYSTEM AT FCB  (FINE CRUSHING BULIDING)

Ready made filter bags along with snap band for leak proof installation of the bag in tube sheet opening. The thickness of tube sheet is 5 mm and a circular opening of 160 mm had been provided in the tube sheet for fixing the filter bag assembly (along with cage and snap band.) the snap band installation shall be capable of leak proof performance even during high pressure air jet cleaning cycle. The bag shall be stitched with 3 rows of thread and its bottom shall be closed with fabric disk & top kept open.

Length of bag				-	5000 mm

MOC					-	Polyester Nonwoven

Thickness of cloth			-	2mm

Weight of fabric			-	not less than 500 gm/sq.meter

(to be mentioned in offer)

Average particle size of dust	-	01 to 05 micron

Average pore size of fabric		-	To be offered by party

Continuous service temperature	-	30-45 º C

Pressure drop during operation	-	minimum 20mm of water column max.

125mm of water column after which it is to be

De-dusted during operation. However party must mention ΔP of the clean bags in the offer.

Quantity 				-  	1000No’s

Party must submit detail drawing with dimension of snap band. & a sample of filter fabric (1sq ft) along with offer for evaluation. Party must give packing details in the offer so that the material can be received in good condition and avoid any damage in transit. One piece filter bag sample will have to be supplied in advance in case of placement of order for approval before bulk supply. Offer without sample (1sq ft) shall be rejected our rightly.